Access Motor CO., LTD

ACCESS AX600U 4x4 EFI Specification

| MODEL | AX600 |
| --- | --- |
| NAME | AX600U 4X4 EFI |
| ENGINE |  |
| Type | 4-stroke, liquid-cooled single; SOHC, 4 valves |
| Cylinder Arrangement | Single cylinder, vertical |
| Displacement | 608cc |
| Bore X Stroke | 96.0 mm ×84.0 mm |
| Fuel Delivery | Fuel Injection (FI), 46mm |
| Compression Ratio | 10.0:1 |
| Ignition | 16-Bit ECU |
| Starting System | Electric |
| Transmission | CVT with reverse , L/H/N/R Gear |
| Final Drive | pushbutton; 3-way locking differential; 2WD, 4WD, locked 4WD; shaft drive |
| CHASSIS |  |
| Suspension/Front | Dual A-arm with 5 way preload-adjustable shocks |
| Suspension/Rear | Dual A-arm with 5 way preload-adjustable shocks |
| Brakes/Front | R/L Disc brake |
| Brakes/Rear | R/L Disc brake |
| Tires/Font | 25x8.00-12 |
| Tires/Rear | 25x10.00-12 |
| DIMENSIONS |  |
| Overall length | 2040 mm |
| Overall width | 1200 mm |
| Overall height | 1252 mm |
| Wheelbase | 1260 mm |
| Ground clearance | 300 mm |
| Claim dry weight | 336 kg |
| Fuel Capacity | 21 L |
| Oil capacity | 2.9 L |
| Battery | GS GT20-BS |
| Lubrication System | Wet sump, Oil Pump |
| Idle Speed | 1500±100 rpm |