Access Motor CO., LTD

SPEED 300S Specification

| Model | SP300 SPORT |
| --- | --- |
| Name | SPEED 300S |
| Engine |  |
| Type | Liquid-cooled, 4 stroke, 2 valve |
| Cylinder arrangement | Single cylinder, vertical |
| Displacement | 280.0 c.c. |
| Bore x Stroke (mm) | Ø74 x 65.2 |
| Carburetor | Mikuni BSR33 |
| Compression Ratio | 11 |
| Ignition | C.D.I. |
| Starting System | Electric |
| Transmission | CVT with reverse , H/L Gear |
| Final drive | Chain/Oil-seal Chain 520 |
| Chassis |  |
| Suspension / Front | Dual A-arm with 5 way preload-adjustable shocks |
| Suspension / Rear | Swing arm with preload adjustable shock |
| Brake / Front | R/L Disc brake |
| Brake / Rear | Disc Brake |
| Tire / Front | AT21 x 7- 10 |
| Tire / Rear | AT20 x 11- 9 |
| Dimensions |  |
| Overall length | 1825 mm |
| Overall width | 1230 mm |
| Overall height | 1140 mm |
| Seat height | 870 mm |
| Wheel base | 1270 mm |
| Ground clearance | 265mm |
| Claimed dry weight | 215 kg |
| Fuel capacity | 14.8 Liter |
| Oil capacity | 1.6 Lit. |
| Battery | GTX12-BS |
| Lubrication system | Wet sump, Oil Pump |
| Idle speed | 1600±100 rpm |